# Lab Report: Conversion of binary sequence into different line encoding

# Objective :

- To understand and implement various digital line encoding schemes (Unipolar, Polar, Bipolar, Manchester, etc.) for a given binary sequence.

- To analyze and compare the signal characteristics of different encoding methods using MATLAB simulation.

# Theory

Line Encoding is the process of converting binary data into a digital signal suitable for transmission over a communication medium.

For a binary data sequence, there are different line encoding schemes, each with its unique characteristics of signal representation, synchronization, and bandwidth usage.

#### Common Line Encoding Schemes:

Unipolar NRZ (Non-Return to Zero):

  - Binary 1 is represented by a high voltage level.

  - Binary 0 is represented by zero voltage.

  - Advantage: Simple implementation.

  - Disadvantage: No self-synchronization; DC component present.

Polar NRZ:

  - Binary 1 is a positive voltage.

  - Binary 0 is a negative voltage.

  - Advantage: Better synchronization than unipolar.

  - Disadvantage: Still has a DC component.

Polar RZ (Return to Zero):

- Binary 1 is a positive voltage for half bit period, then returns to zero.

- Binary 0 is a negative voltage for half bit period, then returns to zero.

- Advantage: Better synchronization.

- Disadvantage: Requires double bandwidth.

Bipolar AMI (Alternate Mark Inversion):

- Binary 0 is represented by zero voltage.

- Binary 1 alternates between positive and negative voltages.

- Advantage: No DC component, better error detection.

- Disadvantage: More complex encoding.

Manchester Encoding:

- Binary 1 is represented by a transition from high to low in the middle of the bit.

- Binary 0 is represented by a transition from low to high in the middle of the bit.

- Advantage: Self-clocking (easy synchronization).

- Disadvantage: Requires double bandwidth.

# 1. Unipolar line encoding

MATLAB Code:

clc;

close all;

clear all;

bits = [1 0 1 1 1 0 1 1];

bitrate = 1;

n = 1000;

T = length(bits)/bitrate;

N = n*length(bits);

dt = T/N;

t = 0:dt:T;

x = zeros(1,length(t));

for i=1:length(bits)

if bits(i)==1

x((i-1)*n+1:(i-1)*n+n/2) = 1;

x((i-1)*n+n/2:i*n) = 1;

else

x((i-1)*n+1:(i-1)*n+n/2) = 0;

x((i-1)*n+n/2:i*n) = 0;

end;

end;

plot(t, x, 'Linewidth', 3);

counter = 0;

for i = 1:length(t)

if t(i)>counter

counter = counter + 1;

if x(i)>0

result(counter) = x(i);

else result(counter) = 0;

end;

end ;

end ;

title('Unipolar linecoding(10111011)/ Nikash / 39');

disp(result);

Output:

2. Polar Line Encoding

Matlab Code:

clc;

close all;

clear all;

bits = [1 1 0 1 1 0 0 1];

bitrate = 1;

n = 1000;

T = length(bits)/bitrate;

N = n*length(bits);

dt = T/N;

t = 0:dt:T;

x = zeros(1,length(t));

for i=1:length(bits)

if bits(i)==1

x((i-1)*n+1:(i-1)*n+n/2) = -1;  x((i-1)*n+n/2:i*n) = -1;

else

x((i-1)*n+1:(i-1)*n+n/2) = 1;  x((i-1)*n+n/2:i*n) = 1;

end;

end;

plot(t, x, 'Linewidth', 3);

counter = 0;

for i = 1:length(t)

if t(i)>counter

counter = counter + 1;

if x(i)>0

result(counter) = x(i);

else result(counter) = 0;

end;

end;

end;

title('Polar(NRZ-L)linecoding (11011001)/ Nikash / 39');

disp(result);

Output:

3. Bipolar Encoding :

Matlab Code:

clc;

clear all;

close all;

bits = [1 0 1 1 0 1 1 0 0 1 0 1];

bitrate = 1;

n = 1000;

T = length(bits)/bitrate;

N = n*length(bits);

dt = T/N;

t = 0:dt:T;

x = zeros(1,length(t));

lastbit = 1;

for i=1:length(bits)

if bits(i)==1

x((i-1)*n+1:i*n) = -lastbit;

lastbit = -lastbit;

end;

end;

plot(t, x, 'Linewidth', 3);

counter = 0;

lastbit = 1;

for i = 1:length(t)

if t(i)>counter

counter = counter + 1;

if x(i)==-lastbit

result(counter) = 1;

lastbit = -lastbit;

else result(counter) = 0;

end;

end;

end;

title('Bipolar Encoding (101101100101)/ Nikash / 39');

disp(result);

Output :

4. Manchester Line Encoding :

Matlab code:

clc;

clear all;

close all;

bits = [1 0 1 0 1 1 0 1];

bitrate = 1;

n = 1000;

T = length(bits)/bitrate;

N = n*length(bits);

dt = T/N;

t = 0:dt:T;

x = zeros(1,length(t));

for i=1:length(bits)

if bits(i)==1

x((i-1)*n+1:(i-1)*n+n/2) = 1;

x((i-1)*n+n/2:i*n) = -1;

else

x((i-1)*n+1:(i-1)*n+n/2) = -1;

x((i-1)*n+n/2:i*n) = 1;

end;

end;

plot(t, x, 'Linewidth', 3);

counter = 0;

for i = 1:length(t)

if t(i)>counter

counter = counter + 1;

if x(i)>0

result(counter) = x(i);

else result(counter) = 0;

end;

end;

end;

title('Manchester line encoding(10101101)/ Nikash / 39 ');

disp(result);

Output:

Discussion

In this lab, we simulated and analyzed different digital line encoding techniques including Unipolar NRZ, Polar NRZ-L, Bipolar AMI, and Manchester encoding using MATLAB. Each encoding method has its unique way of representing binary data over a transmission medium. Unipolar NRZ uses a single positive voltage for logical ‘1’ and zero for ‘0’, making it simple but prone to synchronization problems and containing a DC component. Polar NRZ-L improves by using both positive and negative voltages for logical bits, reducing the DC level but still lacking self-synchronization. Bipolar AMI further enhances performance by alternating the polarity of logical ‘1’s and assigning zero voltage for ‘0’s, thus eliminating the DC component and allowing easier error detection. Manchester encoding stands out for providing reliable synchronization by introducing a transition at the middle of every bit, although it requires more bandwidth. The practical implementation helped visualize the differences in waveform characteristics and signal behavior of each scheme.

## Conclusion

From the lab, it is concluded that while Unipolar and Polar encoding techniques are easier to implement, they are less suitable for reliable data transmission over long distances due to synchronization issues. Bipolar AMI effectively addresses these problems by ensuring no DC component and facilitating error detection. Manchester encoding, despite its higher bandwidth requirement, is highly reliable for synchronization purposes. Therefore, the selection of an appropriate line encoding method should be based on system requirements such as synchronization needs, bandwidth availability, and transmission distance.