# Lab Report: Digital Communication

# Objective :

To understand the basic working principles of analog and digital modulation techniques.

To observe waveform characteristics and understand their time and frequency behavior.

# Theory

### ASK (Amplitude Shift Keying):

- Digital modulation technique where the amplitude of the carrier changes with the binary data.

- 1 → High amplitude, 0 → Zero or lower amplitude.

- Simple but noise-sensitive.

### PSK (Phase Shift Keying):

- Phase of the carrier wave is shifted to represent digital bits.

- Binary PSK (BPSK) uses 0° and 180° phase shifts.

- More robust than ASK in noisy channels.

### FSK (Frequency Shift Keying):

- Frequency of the carrier is changed according to the binary input.

- 1 → f1, 0 → f2 (two distinct frequencies).

- Used in modem communications and wireless transmission.

### PAM (Pulse Amplitude Modulation):

- Analog modulation technique.

- Amplitude of each pulse is proportional to the message signal

- Used in early telephone systems and baseband transmission.

### PWM (Pulse Width Modulation):

- Pulse width (duration) varies according to the message amplitude.

- Common in motor control and power electronics.

- Maintains constant pulse height and spacing.

# 1. ASK (Amplitude Shift Keying):

MATLAB Code:

clc;

clear all;

close all;

Amp = 5;

fp = 4;

fc=50;

t = 0 : 0.001: 1;

x = Amp.*sin(2*pi*fc*t);

subplot(3, 1, 1);

plot(t,x);

grid on;

xlabel('time');

ylabel('amplitude');

title('carrier/ Nikash / 39 ' );

legend("carrier wave");

y = Amp/2.*square(2*pi*fp*t)+(Amp/2);

subplot(3,1,2);

plot(t,y);

grid on;

xlabel('time');

ylabel('amplitude');

legend("message signal");

a = x.*y;

subplot(3,1,3);

plot(t,a);

grid on;

xlabel('angle');

ylabel('amplitude');

title('ASK Signal/ Nikash / 39 ');

legend("ASK signal");

Ask:

2. PSK (Phase Shift Keying)

Matlab Code:

clc;

clear all;

close all;

Amp = 5;

fp = 4;

fc1=50;

fc2=100;

t = 0 : 0.001: 1;

c1 = Amp/2.*sin(2*pi*fc1*t);

c2 = Amp/2.*sin(2*pi*fc2*t);

subplot(4, 1, 1);

plot(t,c1);

grid on;

xlabel('time');

ylabel('amplitude');

title('carrier1/ Nikash / 39 ');

legend("carrier 1 wave");

subplot(4, 1, 2);

plot(t,c2);

grid on;xlabel('time');ylabel('amplitude');

title('carrier2/ Nikash / 39 ');

legend("carrier 2 wave");

m = Amp/2.*square(2*pi*fp*t)+(Amp/2);

subplot(4,1,3);

plot(t,m);

grid on;

xlabel('time');

ylabel('amplitude');

legend("message signal");

for i=0:1000

if m(i+1)==0

a(i+1)=c2(i+1);

else

a(i+1)=c1(i+1);

end

end

subplot(4,1,4);

plot(t,a);

grid on;

xlabel('angle');

ylabel('amplitude');

legend("FSK signal");

PSK (Phase Shift Keying) :

3. FSK (Frequency Shift Keying):

Matlab Code:

clc;

clear all;

close all;

Amp = 5;

fm = 2;

fc=10;

t = 0 : 0.001: 1;

x = Amp.*sin(2*pi*fc*t);

subplot(3, 1, 1);

plot(t,x);

grid on;

xlabel('time');

ylabel('amplitude');

title('carrier/Nikash / 39 ');

legend("carrier wave");

y= square(2*pi*fm*t);

subplot(3,1,2);

plot(t,y);

grid on;

xlabel('time');

ylabel('amplitude');

legend('message signal');

a = x.*y;

subplot(3,1,3);

plot(t,a);

grid on;

xlabel('t');

ylabel('y');

title('PSK/Nikash / 39 ');

legend("PSK signal");

FSK (Frequency Shift Keying):

### 4. PAM (Pulse Amplitude Modulation):

Matlab Code :

clc;

clear all;

close all;

fm= 10; % frequency of the message signal

fc= 100; % frequency of the carrier signal

fs=1000*fm; % (=100KHz) sampling frequency (where 1000 is the upsampling factor)

t=0:1/fs:1; % sampling rate of (1/fs = 100 kHz)

m=1*cos(2*pi*fm*t); % Message signal with period 2*pi*fm (sinusoidal wave signal)

c=0.5*square(2*pi*fc*t)+0.5; % square wave with period 2*pi*fc

s=m.*c; % modulated signal (multiplication of element by element)

subplot(3,1,1);

plot(t,m);

title('Message signal');

xlabel ('Time');

ylabel('Amplitude');

subplot(3,1,2);

plot(t,c);

title('Carrier signal');

xlabel('Time');

ylabel('Amplitude');

subplot(3,1,3);

plot(t,s);

title('Modulated signal');

xlabel('Time');

ylabel('Amplitude');

### PAM (Pulse Amplitude Modulation):

# 5. PWM (Pulse Width Modulation):

MATLAB Code:

clc;

clear all;

close all;

F2=100;

F1=10;

A=5;

t=0:0.001:1;

c=A.*sawtooth(2*pi*F1*t);%Carrier sawtooth

subplot(3,1,1);

plot(t,c);

xlabel('time');

ylabel('Amplitude');

title('Carrier sawtooth wave');

grid on;

m=0.75*A.*sin(2*pi*F2*t);%Message amplitude must be less than Sawtooth

subplot(3,1,2);

plot(t,m);

xlabel('Time');

ylabel('Amplitude');

title('Message Signal');

grid on;

n=length(c);%Length of carrier sawtooth is stored to 'n'

for i=1:n%Comparing Message and Sawtooth amplitudes

if (m(i)>=c(i))

pwm(i)=1;

else

pwm(i)=0;

end

end

subplot(3,1,3);

plot(t,pwm);

xlabel('Time');

ylabel('Amplitude');

title('plot of PWM');

axis([0 1 0 2]);%X-Axis varies from 0 to 1 & Y-Axis from 0 to 2

grid on;

PWM (Pulse Width Modulation):

## Discussion

In this lab, we successfully simulated and observed the working of various modulation techniques including ASK, PSK, FSK, PAM, and PWM. Each method showed unique characteristics in terms of how it modifies the carrier signal to represent digital or analog information. ASK was simple to implement but proved to be more sensitive to noise, making it less reliable in real-world conditions. PSK and FSK, on the other hand, provided better noise immunity, with PSK altering the phase and FSK altering the frequency of the carrier signal based on binary data. PAM and PWM, being analog pulse modulation techniques, showed how continuous signals can be encoded into pulse amplitude and width, respectively. By analyzing the waveforms, we could clearly differentiate the modulation schemes and understand their practical applications and limitations. This helped deepen our understanding of how information is encoded and transmitted over communication systems.

## Conclusion

The lab provided a practical approach to understanding key digital and analog modulation techniques. Through coding and waveform analysis, we were able to clearly visualize how each modulation method represents data. ASK, PSK, and FSK demonstrated the fundamentals of digital modulation, with PSK and FSK offering improved performance in noisy environments. PAM and PWM highlighted important pulse modulation concepts used in analog signal processing and control systems. Overall, the experiment enhanced our conceptual understanding of modulation schemes and their significance in the design and implementation of communication systems.