# Lab Report: Analog Modulation

# Objective

To understand and implement the three types of analog modulation: Amplitude Modulation (AM), Frequency Modulation (FM), and Phase Modulation (PM). Also, to analyze and plot signals for each modulation technique.

# Theory

## Amplitude Modulation

Amplitude Modulation (AM) is a technique used in analog communication where the amplitude of a high-frequency carrier wave is varied in proportion to the instantaneous amplitude of the message signal (baseband signal). The message signal contains the actual information (such as voice or music), while the carrier wave helps transmit it over long distances.

The mathematical representation of AM is:

s(t)=[Ac +m(t)]⋅cos(2πfc t)

Or more simply,

Modulated Signal m(t)=c(t)⋅m(t)

Where:

- c(t) = Carrier signal

- m(t) = Message signal

- Ac  = Amplitude of carrier

- fc  = Carrier frequency

## Why Modulation is Necessary?

### Practical Antenna Size

- Low-frequency signals (like audio signals at 20 kHz) have very long wavelengths.

- The size of the antenna required is proportional to the wavelength (λ = c/f).

- For a 20 kHz signal:

𝜆=𝑐𝑓=3×10820×103=15 km

- Clearly, building such a long antenna is impractical.

- By using a high-frequency carrier, we reduce the required antenna size significantly.

### Narrow Banding of Signa

- AM allows the signal to be confined within a specific frequency band, making it easier to filter, transmit, and receive.

- Without modulation, baseband signals would be prone to interference and poor transmission.

### Multiplexing

  - Modulation allows multiple signals to be transmitted over the same medium simultaneously without interference.

  - Example: In TV broadcasting, video, audio, and control signals are combined and transmitted together using different frequency bands.

# 1. Construct Amplitude Modulation & plot signals for Over modulation, Under modulation & 100% modulation

MATLAB Code:

clc;
close all;
clear all;

fm = input ("Enter message Frequency");
am = input ('Enter message amplitude');
fc = input('Enter Carrier frequency ');
ac = input ('Enter Carrier amplitude');

t = 0:0.001:1;

m = am * sin(2*pi*fm*t);
c = ac * sin(2*pi*fc*t);
x = m.*c;
mod = x + (ac.*c);
subplot(3,1,1);
plot(t,m);
xlabel({'Time (sec) '; 'Fig 1:a'});
ylabel('Amplitude(v)');
title('Message Signal / Nikash / 39 ');
legend('msg');
subplot(3,1,2);
plot(t,c);
xlabel({'Time (sec) '; 'Fig 1:b'});
ylabel('AMP (V) ');
title('Carrier Signal / Nikash / 39 ');
legend('Carrier');
Subplot(3,1,3);
plot(t,mod);
xlabel({'Time (sec)'; 'Fig 1: C'});
ylabel('Amplitude (v)');
title('Amplitude Modulation / Nikash / 39');
legend('AM');

| Modulation Type | Message Frequency (fm) | Message Amplitude (am) | Carrier Frequency (fc) | Carrier Amplitude (ac) |
| --- | --- | --- | --- | --- |
| Over Modulation | 10 Hz | 10 | 100 Hz | 5 |
| 100% Modulation | 10 Hz | 5 | 100 Hz | 10 |
| Under Modulation | 10 Hz | 3 | 100 Hz | 10 |

# Over Modulation

# 100% Modulation

# Under Modulation

# 2. Frequency Modulation

MATLAB Code:

clc;
close all;
clear all;

fm = input ("Enter message Frequency");
am = input ('Enter message amplitude');
fc = input('Enter Carrier frequency ');
ac = input ('Enter Carrier amplitude');
kf = input('Enter Frequency Sensitivity ');

t = 0:0.001:1;

x = am * cos(2*pi*fm*t);
c = ac * cos(2*pi*fc*t);
b = (kf * am)/fm;

A = ac .* cos(2*pi*fc*t + (b.*sin(2*pi*fm*t)));
subplot(3,1,1);
plot(t,x);
xlabel({'Time (sec) '; 'Fig 2:a'});
ylabel('Amplitude(v)');
title('Message Signal / Nikash / 39 ');
legend('msg');
subplot(3,1,2);
plot(t,c);
xlabel({'Time (sec) '; 'Fig 2:b'});
ylabel('AMP (V) ');
title('Carrier Signal / Nikash / 39 ');
legend('Carrier');
Subplot(3,1,3);
plot(t,A);
xlabel({'Time (sec)'; 'Fig 2: C'});
ylabel('Amplitude (v)');
title('Amplitude Modulation / Nikash / 39');
legend('FM');

Input Values:

| Parameter | Value |
| --- | --- |
| Message Frequency (fm) | 25 Hz |
| Message Amplitude (am) | 10 |
| Carrier Frequency (fc) | 400 Hz |
| Carrier Amplitude (ac) | 10 |
| Frequency Sensitivity (kf) | 10 |

Output:

# 3. Phase Modulation

MATLAB Code:

clc;
close all;
clear all;

fm = input ("Enter message Frequency");
am = input ('Enter message amplitude');
fc = input('Enter Carrier frequency ');
ac = input ('Enter Carrier amplitude');
b = input('enter modulation index');

t = 0:0.001:1;

x = am * sin(2*pi*fm*t);
c = ac * sin(2*pi*fc*t);

A = ac.*cos(2*pi*fc*t + (b.*sin(2*pi*fm*t)));
subplot(3,1,1);
plot(t,x);
xlabel({'Time (sec) '; 'Fig 2:a'});
ylabel('Amplitude(v)');
title('Message Signal / Nikash / 39 ');
legend('msg');
subplot(3,1,2);
plot(t,c);
xlabel({'Time (sec) '; 'Fig 2:b'});
ylabel('AMP (V) ');
title('Carrier Signal / Nikash / 39 ');
legend('Carrier');
subplot(3,1,3);
plot(t,A);
xlabel({'Time (sec)'; 'Fig 2: C'});
ylabel('Amplitude (v)');
title('Amplitude Modulation / Nikash / 39');
legend('FM');

Input Values:

| Parameter | Value |
| --- | --- |
| Message Frequency (fm) | 10 Hz |
| Message Amplitude (am) | 10 |
| Carrier Frequency (fc) | 100 Hz |
| Carrier Amplitude (ac) | 10 |
| Modulation Index (b) | 4 |

Output :

## Discussion

In this lab, we implemented and observed three types of analog modulation: AM, FM, and PM using MATLAB. We saw how the message signal affects the carrier wave differently in each method.

- In AM, the amplitude of the carrier changed based on the message.

- In FM, the frequency varied according to the message.

- In PM, the phase of the carrier was modified.

By changing input values, we tested different modulation conditions like under-modulation, over-modulation, and 100% modulation, and visualized their effects through plots.

## Conclusion

The lab successfully demonstrated the implementation and visualization of analog modulation techniques using MATLAB. We understood the mathematical foundation and physical significance of AM, FM, and PM. The experiments showed how signal properties like amplitude, frequency, and phase are affected by different modulation inputs.

This practical understanding is essential in fields like communication systems, broadcasting, and wireless technologies where analog modulation still forms the basis for many real-world applications.